Introduction to Vehicle Fuel Economy Simulation

The objective is to predict the fuel economy (or “miles per gallon,” mpg) of a vehicle under certain driving conditions.  You may know from experience that your car gets better fuel economy driving on the highway than in the city.  This is because the majority of the power output of the engine goes into accelerating the vehicle, while a relatively small fraction goes into propelling your car at a constant speed.  Since most highway driving is at a constant speed, and most city driving is stop and go, the fuel economy of conventionally fueled vehicles is less for city than for highway driving.  In order to standardize the fuel economy figures posted by manufacturers of new cars and trucks, the government has identified drive schedules for highway and city driving.  The schedules are presented as vehicle speed versus time.  For example, the federal urban drive schedule is seen below.

It can be shown that the power to propel a vehicle is given by the following equation:

Wreq=RL+MaV

where Wreq is the power required at the wheels to accelerate the vehicle and overcome drag, rolling resistance, and climbing forces.  The instantaneous vehicle speed is V.  The “road load” is

RL=12ρV2CDA + fW + Wsinθ

where the first, second, and third terms on the right hand side are the aerodynamic drag, rolling resistance, and climbing forces.  The quantity f is termed the rolling resistance coefficient, and is often assumed to remain constant through a drive schedule.

The accelerative load, Ma, includes both the force to increase the vehicle speed and the force necessary to increase the angular speed of rotational components within the vehicle like the driveshaft and transmission components.  Vehicle acceleration can be derived from the speed versus time graph (remember, a = dV/dt).

The question is: how do we connect the power required for a particular drive schedule to fuel economy?  If we neglect the transmission power losses that normally occur between the engine and the wheels, and ignore the engine power needed to overcome internal friction and power vehicle accessories such as lights and fans, we can state that the engine power output must be equal to the power required at the wheels, or

We=Wreq

The internal combustion engine can be modeled as a power cycle where

η = WeQH

From this equation, for a given efficiency, the heat transfer rate in the engine, QH , can be determined.  The mass flow rate of fuel can then be determined from

QH = mf QHV

where QHV is the “heating value” of the fuel expressed in units of energy/mass and  mf is the fuel mass flow rate.  Once the mass flow rate is known, we can divide by fuel density to obtain the volumetric flow rate, V.  The vehicle speed can then be used to determine the instantaneous fuel economy as follows:

FE = VV

A more useful gauge of fuel use is an average fuel economy which can be determined from the ratio of distance driven to fuel volume used:

FEavg=0tfVdt1ρf0tfmfdt