Introduction to Hydraulic Hybrid Vehicles

A hybrid vehicle uses two sources of propulsion in order to conserve energy, thus reducing the consumption of gasoline or diesel fuel.  The addition of an alternate power source does not in and of itself result in improved fuel economy, but if the power distribution between the alternative source and internal combustion engine is controlled so that the engine can run more efficiently, the fuel use can be decreased.  The other benefit of using an alternate power source is that energy from braking can be stored for use during acceleration, energy that would otherwise be wasted in the form of heat generated from friction.  Alternate energy sources/storage systems for hybrid vehicles include motor/generator/batteries, motor/generator/ultracapacitors, motor/generator/flywheels, and hydraulic motor/pump/accumulators.  The hydraulic hybrid is particularly suited for regenerative braking applications.

Hybrid vehicles can be classified into two broad types: series and parallel.  In a series hybrid, the wheels are driven solely by the alternative power source, with the internal combustion engine serving to keep the storage device charged or power the alternate motive source.  There is no direct connection between the IC engine and the wheels.  In a parallel hybrid, both power sources are configured to provide power independently or together directly to the wheels.

engine

fuel tank

shaft power 

alternate power conversion device

energy storage

alternate power delivery/absorption device

Figure 1.  Series hybrid.

fuel tank

energy storage

alternate power delivery/absorption device

engine

Figure 2.  Parallel hybrid.

A hydraulic pump is a device that adds energy from a spinning shaft to a liquid, thereby increasing the liquid pressure.  A hydraulic motor does the opposite: it converts the energy stored in a pressurized liquid into shaft energy.  Hydraulic pump/motor units are designed to operate as either a pump or a motor.  The pressurized liquid is normally stored in an accumulator fitted with a separator (shown in Figure 3 as a piston) between the hydraulic fluid and a compressible gas such as nitrogen.  When the hydraulic unit operates as a pump, it pumps hydraulic fluid from a low pressure reservoir to the accumulator; when it operates as a motor, pressurized fluid flows in the opposite direction, driving the motor and discharging into the low pressure reservoir.

p/m

low pressure reservoir

accumulator

to vehicle drivetrain

hydraulic fluid

N2

Figure 3.  The hydraulic pump/motor unit with accumulator and reservoir.

The parallel hydraulic hybrid, which this paper concentrates on, is illustrated in Figure 4.

driveshaft

engine

transmission

fuel tank

differential

accumulator

p/m unit

reservoir

Figure 4.  Parallel hydraulic hybrid configuration.

The following describes the power split between the IC engine and pump/motor unit.  The MATLAB/Simulink model accounts for inefficiencies in the all components, but for simplicity component efficiencies are neglected in the following description.

The power required at the driveshaft is Wreq, determined by the drive schedule.  Note that Wreq> 0  in power delivery mode,  and  Wreq< 0 in power absorption mode.  In power delivery mode, if Wreq is less than the available power from the hydraulic system, then  Wreq= Wh , where Whis the hydraulic motor power output.  If  Wreq is greater than the pump/motor unit can supply, then the engine augments the pump/motor unit:  Wreq= Wh + We, where We is the engine power.  If the accumulator is empty, then the engine supplies all the power: Wreq= We .

For power absorption mode, the power delivered to the driveshaft by the differential is Wreq= Wh+ Wfric, where now Wh is the power supplied to the pump, and the last term is the power dissipated by friction brakes should the hydraulic pump power capacity be insufficient or the accumulator be full.  If the accumulator is full, Wh= 0.

A characteristic quantity for the accumulator is the state of charge (SOC), defined as

SOC=Va−VVa−Vmin

where Va is the accumulator, or maximum gas, volume, and Vmin is the minimum gas volume.  Note that 0  SOC  1.  The model is initialized by specifying a pre-charge pressure (ppc) and temperature when SOC = 0.  From the IG equation, the mass of gas in the accumulator can then be calculated.