Analysis of the Pump/motor Unit in a Hydraulic Hybrid Vehicle

Names of group members:

In this exercise you will be studying the performance of a pump/motor unit in a hydraulic hybrid vehicle.  When the vehicle brakes, the energy normally dissipated as heat through friction brakes is instead used to drive the pump, forcing hydraulic fluid into an accumulator and pressurizing nitrogen gas.  When the vehicle accelerates, the pressurized hydraulic fluid flows in the reverse direction through the hydraulic unit which now acts as a motor to assist in driving the wheels of the vehicle.  A low pressure reservoir serves as the source of hydraulic fluid when the unit operates in pump mode and as the receiver of hydraulic fluid when the unit operates in motor mode.  See Figure 1 for a schematic of the hydraulic system; Figures 2 and 3 give more detail on the hydraulic pump/motor unit.

p/m

low pressure reservoir

accumulator

to vehicle drivetrain

hydraulic fluid

N2

Figure 1.  Schematic of hydraulic system.  Hydraulic fluid flows through the pump/motor unit from the high-pressure accumulator to the low-pressure reservoir in motor mode and in the opposite direction in pump mode.

Figure 2.  A bent axis variable displacement hydraulic pump/motor unit.  Source:  http://hydraulics.eaton.com/products/pdfs/E-MOPI-MC001-E1.pdf

Figure 3.  Ref:  Basic Fluid Power, D.A. Pease and J.J. Pippenger, 2nd edition, Prentice-Hall, Inc., 1987.

The basic relationships for the pump/motor unit are as follows.  The following equations assume no inefficiencies in components; however, the Simulink model does account for inefficiencies.

The hydraulic pump/motor is modeled as follows.  The pump/motor power is

Wh=Τhωh

Eqn 1

where Τh is the pump/motor torque andωhis the pump/motor angular speed.

From class, you know that the ideal pump/motor power can be determined from mvΔp, or, recognizing that m=Qv,

Wh=ΔpQ

Eqn 2

where Δpis the pressure difference across the pump/motor (paccumulator – preservoir, neglecting losses in hydraulic lines and through fittings) .  With the displacement of the pump/motor, D, defined as the volume of fluid that passes through the unit per radian, we can write the volumetric flow rate as

Q=ωhD

Eqn 3

Comparing equations 1, 2, and 3 it can be seen that the torque is

Τh=ΔpD.

In practice, the pressure in the reservoir is significantly less than that in the accumulator, so we set p =  paccumulator = p in the Simulink model.  The angular speed of the pump/motor shaft is fixed by the transmission reduction ratio from the wheels of the vehicle to the pump/motor unit, and is thus dependent upon the vehicle speed.

The magnitude of the input/output power for pump/motor units used in hydraulic hybrid vehicles is controlled by varying the displacement, D, of the unit.  The displacement is varied in a bent-axis unit (see Figure 3) by adjusting the axis angle.

In practice, the pump/motor power input/output is limited by the maximum displacement of the unit, the available pressure in the accumulator, and the state of charge of the accumulator.  If the state of charge is zero (i.e., the accumulator contains no hydraulic fluid), the hydraulic unit provides no assist to the powertrain.  If the power demand as dictated by the drive schedule is higher than the hydraulic unit can accommodate, the vehicle’s internal combustion engine takes up the difference.  The power absorption capability of the hydraulic system is limited by the same factors; if the state of charge is a hundred percent (i.e., the accumulator can contain no more hydraulic fluid), then the hydraulic system can absorb no more energy, and if the deceleration loads are high enough the friction brakes must make up the difference.

Unlike the steady-state pump problems we’ve seen in class, the hydraulic power unit undergoes highly unsteady operation.  In power mode, as hydraulic fluid leaves the accumulator, the accumulator pressure decreases with time.  The displacement is also constantly changing to meet demand, and the angular speed of the shaft rapidly changes with the vehicle speed (particularly under stop-go conditions like those in the urban drive schedule).  In the following exercises, you will concentrate on analyzing a “snapshot” of the power and performance characteristics of the hydraulic pump/motor at a particular moment in time.

The MATLAB/Simulink model analyzes the performance characteristics of a Class VI delivery truck fitted with a 7.3 liter diesel engine and hydraulic pump/motor system.  The rated power of the diesel engine is 157 kW at 2400 rpm, and the maximum power of the pump/motor unit is 158 kW.  The velocity-time input is from the federal urban drive schedule.  The Simulink model accounts for inefficiencies in all components, engine friction, and simulates a four-speed transmission.

Answer all questions on the following pages, and hand in with the names of your group members listed above.

If you need help using the Scope blocks, right-click on the block and choose “Help.”  To access block options, double-click on the block.

At the end of this exercise, you should be able to:

1)  explain how a bent axis, axial piston pump/motor works

2)  describe how pump/motor displacement and accumulator volume affect hydraulic hybrid fuel economy

3)  change Simulink inputs and compare results

4)  use the scope and display functions in Simulink to analyze output

5)  understand Simulink modeling limitations

1)   Download the Simulink file “hydraulic_hybrid_truck_pump_motor” and input file “input_truck” from the course website and store the files in the “My Documents” folder or on your flashdrive.

2)   Double-click on the file “hydraulic_hybrid_truck_ pump_motor” and wait for MATLAB/Simulink to load.  In the MATLAB workspace, at the >> prompt, type load input_truck to load the input file.

3)  Initially the simulation will use the file “testdrive,” which is a simple constant acceleration, constant speed, constant deceleration drive schedule as seen below:

4)  Run the simulation in the Simulink workspace as explained by your instructor using the default values.

5)  Use the “power and velocity” scope to view the power required, power supplied by the engine, and power supplied by the hydraulic pump/motor unit.  Does the hydraulic system do a good job of capturing braking energy?  What factors limit its ability to capture all the braking energy?  How well does the pump/motor unit assist the engine during acceleration and what limits its ability to provide all the necessary power?  Explain below:

6)  Identify the time when the pump/motor ceases to provide power during the acceleration phase of the test drive.  Write this time and the associated power just prior to this time below.  Print out the plot from the “power and velocity” scope and identify the point with a handwritten arrow and label.  Staple the plot to this handout.

7)  Use scopes to determine the accumulator pressure p, displacement D, and angular speed h at the time identified in the previous step.  Write the values below:

8)  Calculate the motor power using equations 2 and 3 and the values determined in the previous step, and compare to the power read from the plot in step (6).  Discuss the comparison.  Why might there be differences?

9)  The analysis presented here neglects the effects of fluid friction losses in hydraulic hoses transporting fluid from the accumulator to the pump/motor and from the pump/motor to the reservoir.  Explain briefly what would change in the analysis if these effects were included.

10)  Use scopes to determine the hydraulic unit power output, accumulator pressure, and volumetric flowrate at 10 s.  Assuming the hose length from accumulator to the unit is 1 meter, the total fitting loss coefficient, K, is 10, and the hose diameter is 28 mm, estimate the pressure drop from the accumulator to the pump/motor unit.  Is it reasonable to ignore this pressure drop in the Simulink model?

11)  Go to the MATLAB workspace and double the maximum pump/motor displacement, D (the variable name “pmdispl” in the model), in an effort to improve the power delivery and absorption of braking energy.  Run the model again, and look at the “power and velocity” scope.  Is the behavior what you expected?  Look also at the average miles per gallon – or “fuel economy” - (labeled on the display “avg mpg”) for the original and doubled displacement.  Write the mpg values for the original and doubled displacements below.  Is the difference significant?  Why or why not?  What else might you do to improve the hydraulic system performance?

12)  Now increase the accumulator volume (“Va”) by 50 percent.  Are the results better?  Write the results below.

13)  Now change the drive schedule to “uddscolma” (by double-clicking on the “drive schedule” block, double-clicking on the “from workspace” block, and entering uddscolma in the data field) and the simulation time to 1369 s (change the value “37” to “1369” in the text entry box on the top menu bar next to the simulation arrow).  This is the federal urban drive schedule.  Run this drive schedule using both the old and new displacement and accumulator volume, and compare fuel economy.  Write the results below.  Have you improved the performance of the hybrid truck?  What might be some disadvantages to your design changes?

14)  Now increase the fixed time step size in the Simulation/Configuration Parameters dialog box from 0.5 to 1 second.  Run the simulation and compare to fuel economy results from (13).  Are the results significantly different?  Did the length of time to run the simulation change?  Which time step do you think results in the most accurate model?  Change the time step to 0.1 s and discuss the results.