Hydraulic Hybrid Simulation

Power Analysis

Names of group members:

answer key

In this exercise you will analyze the energy and power characteristics of a hydraulic hybrid vehicle for an urban drive schedule.  You will be using the following Simulink blocks:  Scope, Display, Integrator, Constant, Product, and Saturation.  These blocks can be found in the Simulink library, accessed in the View/Library Browser menu in the Simulink workspace.  If you need help using these blocks, right-click on the block and choose “Help.”  To access block options, double-click on the block.

At the end of this exercise, you should be able to:

1)  describe the relationship between power and energy

2)  describe how a hydraulic hybrid powertrain reduces fuel consumption

3)  use MATLAB/Simulink to integrate rate quantities and manipulate outputs

4)  use the scope and display functions in Simulink to analyze output

5)  use the “Help” function in Simulink to learn how to perform operations

The MATLAB/Simulink model analyzes the performance characteristics of a Class VI delivery truck fitted with a 7.3 liter diesel engine and hydraulic pump/motor system.  The rated power of the diesel engine is 157 kW at 2400 rpm, and the maximum power of the pump/motor unit is 158 kW.  The velocity-time input is from the federal urban drive schedule.  The Simulink model accounts for inefficiencies in all components, engine friction, and simulates a four-speed transmission.

Answer all questions on the following pages, and hand in with the names of your group members listed above.

1)   Download the Simulink file “hydraulic_hybrid_truck_power” and input file “input_truck” from the course website and store the files in the “My Documents” folder or on your flashdrive.

2)   Double-click on the file “hydraulic_hybrid_truck_power” and wait for MATLAB/Simulink to load.  In the MATLAB workspace, at the >> prompt, type load input_truck to load the input file.

3)   In the Simulink workspace, identify the manual switch that turns the Pump/Motor unit on and off (see the figure at the end of this handout).

4)   Run the simulation as explained by your instructor using the default values, with the hydraulic pump/motor unit off.  Use the existing scope labeled “power and velocity” to view the required power (yellow), engine power (purple), and hydraulic power (blue) values.  Use the binoculars button to fill the scope with the plot for the entire simulation.  Zoom in on the time period from 400 s to 600 s.  Is the behavior what you would expect?  Explain below.  Print the plots and hand-write the appropriate labels.  Staple the plots to this handout.

The upper plot shows power in watts.  The purple (engine) and yellow (required power) coincide because the hydraulic system is off (the blue line representing the hydraulic power remains at zero throughout the drive).  The curve above the x-axis represents the power required for driving, and the curve below the x-axis represents the power dissipated during braking.  Comparing the velocity (lower) plot to the power plot, it can be seen that high power is required for acceleration, relatively low power is required for constant driving speed, and negative power is generated during deceleration.

5)   Now run the simulation with the pump/motor on.  Use the existing scope labeled “power and velocity” to view the required power (yellow), engine power (purple), and hydraulic power (blue) values.  Zoom in on the time period from 400 s to 600 s.  Is the behavior what you would expect?  Explain below.  Print the plots and hand-write the appropriate labels.  Staple the plots to this handout.

During the periods when power is demanded at the wheels (positive power as indicated by the yellow curve), the hydraulic system provides part of the power, thus decreasing the required engine power.  At certain points, the hydraulic power suddenly decreases because the accumulator contains no more hydraulic fluid, and the engine thus must provide all of the required power.  During braking, the hydraulic system absorbs a portion of the braking power, but not all.  Limited pump displacement and accumulator volume serve to limit the amount of braking power absorbed.  The distance between the yellow and blue lines in the negative power region represents braking power dissipated as heat by the friction brakes.

6)   What does the area under these power curves represent?  Does the pump/motor do a good job of capturing braking energy?  How well does it assist the engine during acceleration?  Explain below.

The area between the curves and the x-axis represents energy.  During periods of negative required power (braking), the hydraulic system captures some, but not all, of the braking energy.  From the figure the hydraulic system captures roughly from 50 to 75 percent of the braking energy.  During periods of positive required power, the hydraulic system appears to provide less than 50 percent of the required power until the accumulator is depleted; at this point the engine assumes the entire load.

7)   Use the Simulink integrator function to calculate the a) energy potentially stored from braking, and b) energy required for driving.  Note that you will have to use the “saturation” block to isolate negative (braking, power from the wheels) and positive (power to the wheels) values for Wreq.  Display the results using the “display” block.  What percentage of the driving energy could be provided by stored braking energy?  Write your answer below:

The values for driving and braking energy are 3.68  107 joules and 1.34  107 joules, respectively, resulting in 36.4 percent of the driving energy potentially being supplied by captured braking energy.  In the figure below, the saturation block preceding the driving energy display captures only positive power values; the saturation block preceding the braking energy block captures only negative power values.

8)   Use the Simulink integrator function to calculate the a) energy absorbed (during braking) by the pump/motor unit, and b) energy required for driving.  What percentage of the driving energy could be provided by stored braking energy?  Write your answer below:

8.69   106 joules was absorbed by the hydraulic accumulator.  Thus, 8.69   106/3.68  107 = 23.6 percent of the driving energy could be supplied by the hydraulic system.  In the figure below, the saturation block preceding the energy absorbed display captures only the negative hydraulic power values.

9)   Use the Simulink integrator function to calculate the a) energy provided by the pump/motor unit, and b) energy required for driving.  What percentage of the driving energy was provided by stored braking energy?  What is the difference between this answer and that from (8)?  Write your answers below:

6.88   106 joules was provided by the hydraulic system, resulting in 6.88   106/3.68  107 = 18.7 percent provided by the hydraulic system.  The answer is less than the 23.6 percent calculated in (8) due to inefficiencies in the pump/motor system and transmission losses.  The power provided by the hydraulic system was captured with a saturation block isolating the positive hydraulic power values.

10) Use the Simulink integrator function and appropriate math functions in conjunction with the fuel mass flow rate output to determine the number of gallons used for the drive.  Also determine the distance traveled by integrating the velocity, and convert to miles.  Run the model with and without the hydraulic system on and write down the fuel economy in miles per gallon (mpg) for the IC engine only and hybrid powertrain below:

Miles/gallon for IC engine only:

10.91

Miles/gallon for hybrid:

12.67

What is the percentage improvement in fuel economy when using the hybrid powertrain?

16 percent

11)  In the MATLAB workspace, change the lower heating value, QLHV (note that MATLAB is case sensitive!), from 43  106 J/kg (this is the energy content of regular diesel fuel) to 38.5  106 J/kg (this is the energy content of biodiesel).  Compare the fuel economy to that from (10).  Do the results make sense?  Describe the comparison below:

11.34 mpg with biodiesel vs. 12.67 mpg for regular diesel, a 10.5 percent decrease.  This makes sense because the biodiesel releases about 10.5 percent less energy than the regular diesel.

on/off switch for pump/motor unit

fuel mass flow rate

velocity